/*

* ESP32 Smart System - Optimized Version

* Non-Blocking | Batch HTTP Requests | FreeRTOS Tasks

* With Supabase and Blynk

*/

/* ================= BLYNK CONFIG (MUST BE FIRST!) ================= */

#define BLYNK_TEMPLATE_ID   "TMPL21_0J5RRM"

#define BLYNK_TEMPLATE_NAME "Smart City IOT SystemCopy"

#define BLYNK_AUTH_TOKEN    "J95HUNtrZRojrICq7_a4b8WNXtNTuMrj"

#define BLYNK_PRINT Serial

/* ================= INCLUDES ================= */

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include <SPI.h>

#include <MFRC522.h>

#include <BlynkSimpleEsp32.h>

#include <time.h>

#include <Preferences.h>

/* ================= DEBUG MODE ================= */

#define DEBUG_MODE 1  // Set to 0 to disable verbose logging

/* ================= WiFi CONFIG ================= */

const char* WIFI_SSID     = "5enoo";

const char* WIFI_PASSWORD = "5enoooooo1";

/* ================= SUPABASE CONFIG ================= */

const String SUPABASE_URL = "https://ezpflxkgwemymisdgkoe.supabase.co";

const String SUPABASE_KEY = "eyJhbGciOiJIUzI1NiIsInR5cCI6IkpXVCJ9.eyJpc3MiOiJzdXBhYmFzZSIsInJlZiI6ImV6cGZseGtnd2VteW1pc2Rna29lIiwicm9sZSI6ImFub24iLCJpYXQiOjE3NzY5NDQ3MDUsImV4cCI6MjA5MjUyMDcwNX0.MKFErh-QDACfAx5797AjvlIdA8ftKD6HWrvq-6LLO2w";

const String DEVICE_ID    = "esp32_main_01";

/* ================= PINS ================= */

// RFID

#define RFID_SS_PIN  5

#define RFID_RST_PIN 22

// IR & LDR

#define IR1_PIN 34

#define IR2_PIN 35

#define LDR_PIN 32

// LEDs

#define GATE_LED  14

#define GATE_LED2 2

#define PWM_LED_1 12

#define PWM_LED_2 13

// Soil

#define SOIL_PIN 33

// PWM Settings

#define PWM_FREQ       5000

#define PWM_RESOLUTION 8

/* ================= BLYNK VIRTUAL PINS ================= */

#define VPIN_RFID_UID       V0

#define VPIN_RFID_COUNT     V1

#define VPIN_RFID_LED       V2

#define VPIN_LDR_VALUE      V3

#define VPIN_IR1_STATUS     V4

#define VPIN_IR2_STATUS     V5

#define VPIN_LIGHT_CONTROL  V6

#define VPIN_SOIL_MOISTURE  V7

#define VPIN_BRIGHTNESS_1   V8

#define VPIN_BRIGHTNESS_2   V9

#define VPIN_DEVICE_ONLINE  V10

/* ================= OBJECTS ================= */

MFRC522 rfid(RFID_SS_PIN, RFID_RST_PIN);

BlynkTimer timer;

Preferences preferences;

/* ================= RFID VARS ================= */

const String ACCEPTED_UIDS[] = {"A7CA1EAF"};

const int    ACCEPTED_COUNT  = 1;

String       lastUID         = "none";

bool         lastAccessGranted = false;  // Track access control status

unsigned long rfidAccessCount = 0;

/* ================= SENSOR VARS ================= */

const int LDR_THRESHOLD = 2000;

unsigned long lastMotion1Time = 0;

unsigned long lastMotion2Time = 0;

const unsigned long LIGHT_DELAY        = 3000;

const unsigned long DB_SEND_INTERVAL   = 10000;

const unsigned long BLYNK_UPDATE_INTERVAL = 3000; // Reduced frequency

unsigned long lastDBSend = 0;

int   ldrValue        = 0;

bool  ir1Detected     = false;

bool  ir2Detected     = false;

int   brightness1     = 0;

int   brightness2     = 0;

int   soilValue       = 0;

float soilMoisturePct = 0;

int   lightControlMode = 0; // 0=Auto, 1=Force OFF, 2=Force ON

/* ================= NON-BLOCKING RFID VARS ================= */

unsigned long rfidLedStartTime = 0;

bool rfidLedActive = false;

bool rfidDeniedBlinkState = false;

int rfidDeniedBlinkCount = 0;

unsigned long rfidDeniedBlinkTime = 0;

/* ================= BATCH DATA STRUCTURE ================= */

struct SensorBatch {

String rfid;

bool accessGranted;  // Access control status

int ldr;

bool ir1;

bool ir2;

float soil;

int brightness1;

int brightness2;

unsigned long rfidCount;

String timestamp;

};

SensorBatch currentBatch;

/* ================= TASK HANDLES ================= */

TaskHandle_t httpTaskHandle = NULL;

SemaphoreHandle_t dataMutex = NULL;

volatile bool dataReadyToSend = false;

/* ================= FUNCTION DECLARATIONS ================= */

void handleRFID();

void handleLighting();

void handleSoil();

void updateBlynkDashboard();

void sendBatchToSupabase();

void httpTask(void* parameter);

String getTimestamp();

bool checkUID(String uid);

void debugPrint(String msg);

/* ================= DEBUG HELPER ================= */

void debugPrint(String msg) {

#if DEBUG_MODE

Serial.println(msg);

#endif

}

/* ================= SETUP ================= */

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\n========================================");

Serial.println("  ESP32 Smart System - OPTIMIZED");

Serial.println("  Non-Blocking | Batch Requests");

Serial.println("========================================\n");

// GPIO

pinMode(IR1_PIN, INPUT);

pinMode(IR2_PIN, INPUT);

pinMode(GATE_LED, OUTPUT);

pinMode(GATE_LED2, OUTPUT);

digitalWrite(GATE_LED, LOW);

digitalWrite(GATE_LED2, LOW);

// PWM

ledcAttach(PWM_LED_1, PWM_FREQ, PWM_RESOLUTION);

ledcAttach(PWM_LED_2, PWM_FREQ, PWM_RESOLUTION);

// RFID

SPI.begin();

rfid.PCD_Init();

Serial.println("✓ RFID initialized");

// Load saved RFID count

preferences.begin("smart_city", false);

rfidAccessCount = preferences.getULong("rfid_count", 0);

Serial.println("✓ RFID access count loaded: " + String(rfidAccessCount));

// Create Mutex for thread-safe data access

dataMutex = xSemaphoreCreateMutex();

// WiFi

Serial.print("Connecting to WiFi");

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 30) {

delay(500);

Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\n✓ WiFi Connected: " + WiFi.localIP().toString());

Blynk.config(BLYNK_AUTH_TOKEN);

Blynk.connect();

if (Blynk.connected()) {

Serial.println("✓ Blynk Connected");

Blynk.virtualWrite(VPIN_DEVICE_ONLINE, 1);

Blynk.virtualWrite(VPIN_RFID_COUNT, rfidAccessCount);

Blynk.virtualWrite(VPIN_LIGHT_CONTROL, lightControlMode);

}

// Create HTTP Task on Core 0 (Core 1 runs loop())

xTaskCreatePinnedToCore(

httpTask,           // Task function

"HTTP_Task",        // Task name

8192,               // Stack size (bytes)

NULL,               // Parameters

1,                  // Priority

&httpTaskHandle,    // Task handle

0                   // Core 0

);

Serial.println("✓ HTTP Task created on Core 0");

} else {

Serial.println("\n✗ WiFi Failed - Running Offline");

}

// NTP

configTime(0, 0, "pool.ntp.org");

// Blynk timer

timer.setInterval(BLYNK_UPDATE_INTERVAL, updateBlynkDashboard);

Serial.println("\n=== SMART SYSTEM READY ===");

Serial.println("==========================\n");

}

/* ================= RFID (NON-BLOCKING) ================= */

bool checkUID(String uid) {

for (int i = 0; i < ACCEPTED_COUNT; i++) {

if (uid.equals(ACCEPTED_UIDS[i])) return true;

}

return false;

}

void handleRFID() {

// Handle non-blocking LED states first

unsigned long now = millis();

// Handle granted LED timeout

if (rfidLedActive && (now - rfidLedStartTime >= 1000)) {

digitalWrite(GATE_LED, LOW);

rfidLedActive = false;

if (Blynk.connected()) {

Blynk.virtualWrite(VPIN_RFID_LED, LOW);

}

}

// Handle denied LED blinking

if (rfidDeniedBlinkCount > 0) {

if (now - rfidDeniedBlinkTime >= 300) {

rfidDeniedBlinkState = !rfidDeniedBlinkState;

digitalWrite(GATE_LED2, rfidDeniedBlinkState ? HIGH : LOW);

rfidDeniedBlinkTime = now;

if (!rfidDeniedBlinkState) {

rfidDeniedBlinkCount--;

}

}

return; // Don't check for new cards while blinking

}

// Check for new RFID card

if (!rfid.PICC_IsNewCardPresent()) return;

if (!rfid.PICC_ReadCardSerial()) return;

String uid = "";

for (byte i = 0; i < rfid.uid.size; i++) {

if (rfid.uid.uidByte[i] < 0x10) uid += "0";

uid += String(rfid.uid.uidByte[i], HEX);

}

uid.toUpperCase();

uid.trim();

lastUID = uid;

debugPrint("----- RFID: " + uid + " -----");

if (checkUID(uid)) {

rfidAccessCount++;

preferences.putULong("rfid_count", rfidAccessCount);

debugPrint("✓ Access Granted | Count: " + String(rfidAccessCount));

// Set access granted status

lastAccessGranted = true;

// Turn on LED (non-blocking)

digitalWrite(GATE_LED, HIGH);

rfidLedActive = true;

rfidLedStartTime = now;

// Update Blynk immediately

if (Blynk.connected()) {

Blynk.virtualWrite(VPIN_RFID_UID, uid);

Blynk.virtualWrite(VPIN_RFID_COUNT, rfidAccessCount);

Blynk.virtualWrite(VPIN_RFID_LED, HIGH);

}

} else {

debugPrint("✗ Access Denied");

// Set access denied status

lastAccessGranted = false;

// Start blinking sequence (non-blocking)

rfidDeniedBlinkCount = 4; // 2 blinks = 4 state changes

rfidDeniedBlinkTime = now;

rfidDeniedBlinkState = true;

digitalWrite(GATE_LED2, HIGH);

}

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

}

/* ================= LIGHTING (PWM) ================= */

void handleLighting() {

ir1Detected = digitalRead(IR1_PIN) == HIGH;

ir2Detected = digitalRead(IR2_PIN) == HIGH;

ldrValue = analogRead(LDR_PIN);

bool isDark = ldrValue > LDR_THRESHOLD;

unsigned long now = millis();

if (ir1Detected) lastMotion1Time = now;

if (ir2Detected) lastMotion2Time = now;

if (lightControlMode == 1) {

brightness1 = 0;

brightness2 = 0;

} else if (lightControlMode == 2) {

brightness1 = 255;

brightness2 = 255;

} else {

brightness1 = 0;

brightness2 = 0;

if (isDark && (now - lastMotion1Time < LIGHT_DELAY)) {

brightness1 = map(ldrValue, 2000, 4095, 50, 255);

}

if (isDark && (now - lastMotion2Time < LIGHT_DELAY)) {

brightness2 = map(ldrValue, 2000, 4095, 50, 255);

}

}

ledcWrite(PWM_LED_1, brightness1);

ledcWrite(PWM_LED_2, brightness2);

}

/* ================= SOIL ================= */

void handleSoil() {

soilValue = analogRead(SOIL_PIN);

soilMoisturePct = map(soilValue, 4095, 0, 0, 100);

soilMoisturePct = constrain(soilMoisturePct, 0, 100);

}

/* ================= BLYNK DASHBOARD (OPTIMIZED) ================= */

void updateBlynkDashboard() {

if (!Blynk.connected()) return;

// Use Blynk.virtualWrite for individual updates (already optimized by Blynk library)

Blynk.virtualWrite(VPIN_RFID_UID,      lastUID);

Blynk.virtualWrite(VPIN_RFID_COUNT,    rfidAccessCount);

Blynk.virtualWrite(VPIN_LDR_VALUE,     ldrValue);

Blynk.virtualWrite(VPIN_IR1_STATUS,    ir1Detected ? "Motion" : "Clear");

Blynk.virtualWrite(VPIN_IR2_STATUS,    ir2Detected ? "Motion" : "Clear");

Blynk.virtualWrite(VPIN_SOIL_MOISTURE, (int)soilMoisturePct);

Blynk.virtualWrite(VPIN_BRIGHTNESS_1,  brightness1);

Blynk.virtualWrite(VPIN_BRIGHTNESS_2,  brightness2);

// Device online is sent less frequently to reduce traffic

static unsigned long lastOnlineUpdate = 0;

if (millis() - lastOnlineUpdate > 30000) { // Every 30 seconds

Blynk.virtualWrite(VPIN_DEVICE_ONLINE, 1);

lastOnlineUpdate = millis();

}

}

/* ================= BLYNK LIGHT CONTROL ================= */

BLYNK_WRITE(VPIN_LIGHT_CONTROL) {

lightControlMode = param.asInt();

debugPrint("Light Mode: " + String(lightControlMode));

}

/* ================= BLYNK CONNECTED ================= */

BLYNK_CONNECTED() {

Serial.println("✓ Blynk reconnected");

Blynk.virtualWrite(VPIN_DEVICE_ONLINE, 1);

Blynk.virtualWrite(VPIN_RFID_COUNT, rfidAccessCount);

Blynk.syncVirtual(VPIN_LIGHT_CONTROL);

}

/* ================= SUPABASE (BATCH REQUEST) ================= */

void sendBatchToSupabase() {

// Prepare batch data with mutex protection

if (xSemaphoreTake(dataMutex, portMAX_DELAY)) {

currentBatch.rfid = lastUID;

currentBatch.accessGranted = lastAccessGranted;

currentBatch.ldr = ldrValue;

currentBatch.ir1 = ir1Detected;

currentBatch.ir2 = ir2Detected;

currentBatch.soil = soilMoisturePct;

currentBatch.brightness1 = brightness1;

currentBatch.brightness2 = brightness2;

currentBatch.rfidCount = rfidAccessCount;

currentBatch.timestamp = getTimestamp();

dataReadyToSend = true;

xSemaphoreGive(dataMutex);

}

}

/* ================= HTTP TASK (RUNS ON CORE 0) ================= */

void httpTask(void* parameter) {

while (true) {

if (dataReadyToSend && WiFi.status() == WL_CONNECTED) {

// Copy data with mutex protection

SensorBatch localBatch;

if (xSemaphoreTake(dataMutex, portMAX_DELAY)) {

localBatch = currentBatch;

dataReadyToSend = false;

xSemaphoreGive(dataMutex);

}

// Send batch request (single HTTP call)

HTTPClient http;

http.setTimeout(5000);

http.begin(SUPABASE_URL + "/rest/v1/sensor_readings");

http.addHeader("apikey",        SUPABASE_KEY);

http.addHeader("Authorization", "Bearer " + SUPABASE_KEY);

http.addHeader("Content-Type",  "application/json");

http.addHeader("Prefer",        "return=minimal");

// Create JSON with all sensor data in ONE request

DynamicJsonDocument doc(512);

doc["device_id"]   = DEVICE_ID;

doc["timestamp"]   = localBatch.timestamp;

doc["rfid_uid"]    = localBatch.rfid;

doc["access_granted"] = localBatch.accessGranted;

doc["rfid_count"]  = localBatch.rfidCount;

doc["ldr_value"]   = localBatch.ldr;

doc["ir1_motion"]  = localBatch.ir1;

doc["ir2_motion"]  = localBatch.ir2;

doc["soil_moisture"] = localBatch.soil;

doc["brightness1"] = localBatch.brightness1;

doc["brightness2"] = localBatch.brightness2;

String json;

serializeJson(doc, json);

int httpCode = http.POST(json);

if (httpCode == 200 || httpCode == 201 || httpCode == 204) {

debugPrint("✓ Batch sent to Supabase");

} else {

Serial.println("✗ Supabase failed (HTTP " + String(httpCode) + ")");

}

http.end();

}

vTaskDelay(100 / portTICK_PERIOD_MS); // Check every 100ms

}

}

/* ================= TIMESTAMP ================= */

String getTimestamp() {

struct tm timeinfo;

if (!getLocalTime(&timeinfo)) return String(millis());

char buffer[25];

strftime(buffer, sizeof(buffer), "%Y-%m-%dT%H:%M:%SZ", &timeinfo);

return String(buffer);

}

/* ================= LOOP (NON-BLOCKING) ================= */

void loop() {

if (WiFi.status() == WL_CONNECTED) {

Blynk.run();

}

timer.run();

handleRFID();      // Non-blocking now

handleLighting();  // Fast sensor reads

handleSoil();      // Fast sensor reads

// Trigger batch send every 10 seconds

if (millis() - lastDBSend > DB_SEND_INTERVAL) {

if (WiFi.status() == WL_CONNECTED) {

sendBatchToSupabase(); // Prepares data, HTTP task sends it

}

lastDBSend = millis();

}

// No delay() - fully non-blocking!

}